Annotated Code Document

| Code | Annotations |
| --- | --- |
| import pandas as pd import numpy as np import matplotlib.pyplot as plt import seaborn as sns sns.set_style('whitegrid') sns.set_context('notebook') | These lines load an external Python library so its functions may be used in the project. The final two lines apply a style and color palette to the figures. |
| nhts_data = pd.read_csv('NHTS.csv') ngsim_data = pd.read_csv('NGSIM.csv') nhts_data.columns ngsim_data.columns | This section loads the two CSV files into Python and prints the column names of each dataset. The columns function lets the user see what each column is labeled for further use. |
| vehicle_counts = vehicle_type_data['vehicle_type'].value_counts() plt.figure(figsize=(10, 6)) vehicle_counts.plot(kind='bar', edgecolor='black') plt.title('Count of Vehicles by Vehicle Type') plt.xlabel('Vehicle Type') plt.ylabel('Count') plt.show() | This section creates the required bar chart. The value_counts() function counts how many times each vehicle type appears in the dataset, and the bar chart displays the counts. |
| plt.figure(figsize=(10, 6)) plt.hist(vehicle_age_data['vehicle_age'], bins=20, edgecolor='black') plt.title('Distribution of Vehicle Age') plt.xlabel('Vehicle Age (years)') plt.ylabel('Frequency') plt.show() | This section creates the histogram for vehicle_age. A histogram is used here because it shows the distribution of one numerical variable and helps show whether most vehicles are newer or older. |
| plt.figure(figsize=(8, 6)) sns.boxplot(data=boxplot_data, x='household_location', y='vehicle_age') plt.title('Vehicle Age by Household Location') plt.xlabel('Household Location') plt.ylabel('Vehicle Age (years)') plt.show() | This code creates the required boxplot. It compares the distribution of vehicle_age across household_location, which shows the difference in the spread and average vehicle age for each group. |
| plt.figure(figsize=(10, 6)) vehicle_counts.plot(kind='bar', color='maroon', edgecolor='black') plt.title('Customized Vehicle Type Bar Chart', fontsize=18) plt.xticks(rotation=45, ha='right') plt.show() | This graph is the customized plot required by the client prompt. The code changes the bar color, font size, and x-axis label rotation, which shows that the group can modify the base plot format. |
| trajectory_number = 2 data_subset = ngsim_data[ ngsim_data['trajectory_number'] == trajectory_number ].copy() | This section uses a selected trajectory number so the time series plots and simulation can be based on one clear leader-follower pair. |
| sns.lineplot(data=data_subset, x='Time', y='leader_position(m)', label='Leader Position') sns.lineplot(data=data_subset, x='Time', y='follower_position(m)', label='Follower Position') | These lines are used for the first NGSIM time-series plot. They show leader position and follower position versus time so the user can see how the two vehicles move through the selected trajectory. |
| sns.lineplot(data=data_subset, x='Time', y='leader_speed(m/s)', label='Leader Speed') sns.lineplot(data=data_subset, x='Time', y='follower_speed(m/s)', label='Follower Speed') | These lines are used for the second NGSIM time-series plot. They compare leader speed and follower speed versus time, which helps show how the follower responds to the leader’s movement. |
| def idm_acceleration(v, s, delta_v, v0, s0, T, a, b, delta): s_star = s0 + v * T + (v * delta_v) / (2 * np.sqrt(a * b)) s_star = max(s_star, s0) s = max(s, 0.1) acceleration = a * (1 - (v / v0) ** delta - (s_star / s) ** 2) return acceleration | This function defines the Intelligent Driver Model (IDM) acceleration equation. It uses follower speed, the spacing between vehicles, and the speed difference to calculate the follower’s next acceleration value. |
| time_data = data_subset['Time'].values leader_position = data_subset['leader_position(m)'].values leader_speed = data_subset['leader_speed(m/s)'].values real_follower_position = data_subset['follower_position(m)'].values real_follower_speed = data_subset['follower_speed(m/s)'].values leader_acceleration = data_subset['leader_acc(m/s^2)'].values real_follower_acceleration = data_subset['follower_acc(m/s^2)'].values | This section extracts the needed NGSIM columns into arrays. These arrays are used later in the simulation loop and in the final comparison plots. |
| v0 = 30 s0 = 2 T = 1.5 a_param = 1.0 b = 1.5 delta = 4 sim_position = np.zeros(n_steps) sim_speed = np.zeros(n_steps) sim_acceleration = np.zeros(n_steps) | These lines set the IDM parameters and create empty arrays for the simulated follower position, speed, and acceleration. The simulation starts from the real follower’s first position and speed. |
| for i in range(n_steps - 1): gap = max(leader_position[i] - sim_position[i], 0.1) delta_v = sim_speed[i] - leader_speed[i] sim_acceleration[i] = idm_acceleration( sim_speed[i], gap, delta_v, v0, s0, T, a_param, b, delta ) sim_speed[i + 1] = max(0, sim_speed[i] + sim_acceleration[i] * dt) sim_position[i + 1] = sim_position[i] + sim_speed[i] * dt | This loop runs the IDM simulation. At each time step, it computes the simulated acceleration using the IDM function and updates the follower’s speed and position. |
| plt.plot(time_data, leader_acceleration, label='Leader Acceleration') plt.plot(time_data, real_follower_acceleration, label='Real Follower Acceleration') plt.plot(time_data, sim_acceleration, label='Simulated Follower Acceleration', linestyle='--') plt.legend() plt.show() | This is the main comparison plot required in the client prompt. It places the leader acceleration, the real follower acceleration, and the simulated follower acceleration on the same graph so the group can compare real and modeled behavior. |
| plt.figure(figsize=(12, 6)) plt.plot(time_data, real_follower_position, label='Real Follower Position') plt.plot(time_data, sim_position, label='Simulated Follower Position', linestyle='--') plt.plot(time_data, leader_position, label='Leader Position', linestyle=':') plt.title(f'Position Comparison Using IDM (Trajectory {trajectory_number})', fontsize=16) plt.xlabel('Time (s)', fontsize=14) plt.ylabel('Position (m)', fontsize=14) plt.legend() plt.grid(True) plt.tight_layout() plt.show() | This final plot compares the real follower position, simulated follower position, and leader position over time. It helps show how closely the IDM simulation follows the real NGSIM trajectory. |